Сеть

| Устройство | Интерфейс | IP-адрес | Маска | VLAN | Подсеть | Шлюз |
| --- | --- | --- | --- | --- | --- | --- |
| ISP | enp7s1 (Internet) | DHCP | DHCP | - | DHCP | DHCP |
|  | enp7s2 (to HQ-RTR) | 172.16.1.1 | 28 | - | 172.16.1.0 | - |
|  | enp7s3 (to BR-RTR) | 172.16.2.1 | 28 | - | 172.16.2.0 | - |
| HQ-RTR | enp7s1 (to ISP) | 172.16.1.2 | 28 | - | 172.16.1.0 | 172.16.1.1 |
|  | enp7s2.100 (to HQ-SRV) | 192.168.100.1 | 29 | 100 | 192.168.100.0 | - |
|  | enp7s2.200 (to HQ-CLI) | 192.168.200.1 | 24 | 200 | 192.168.200.0 | - |
|  | enp7s2.999 (для админ.) | 192.168.99.1 | 29 | 999 | 192.168.99.0 | - |
|  | gre1 (IP туннель) | 192.168.255.1 | 30 | - | 192.168.255.0 | - |
| BR-RTR | enp7s1 (to ISP) | 172.16.200.2 | 28 | - | 172.16.2.0 | 172.16.200.1 |
|  | enp7s2 (to BR-SRV) | 192.168.3.1 | 29 | - | 192.168.3.0 | - |
|  | gre1 (IP туннель) | 192.168.255.2 | 30 | - | 192.168.255.0 | - |
| HQ-SRV | enp7s1.100 (to HQ-RTR) | 192.168.1.2 | 29 | 100 | 192.168.1.0 | 192.168.100.1 |
| BR-SRV | enp7s1 (to BR-RTR) | 192.168.3.2 | 29 | - | 192.168.3.0 | 192.168.3.1 |
| HQ-CLI | enp7s1.200 (to HQ-RTR) | (DHCP) 192.168.200.2 | (DHCP) 24 | 200 | 192.168.200.0 | 192.168.200.1 |

ISP:

# Имя хоста

# https://www.altlinux.org/Настройка_сети

hostnamectl hostname isp.au-team.irpo

exec bash

# Интерфейсы

cd /etc/net/ifaces

mkdir enp7s2 enp7s3

echo TYPE=eth > enp7s2/options

echo TYPE=eth > enp7s3/options

echo 172.16.1.1/28 > enp7s2/ipv4address

echo 172.16.2.1/28 > enp7s3/ipv4address

systemctl restart network

# NAT

# https://habr.com/ru/sandbox/274318/

apt-get update

apt-get install -y nano

apt-get install -y iptables

nano /etc/net/sysctl.conf

net.ipv4.ip_forward = 1

sysctl -p /etc/net/sysctl.conf

iptables -t nat -A POSTROUTING -o enp7s1 -j MASQUERADE

iptables-save > /etc/sysconfig/iptables

systemctl enable --now iptables

# Часовой пояс

apt-get install tzdata -y

timedatectl set-timezone Europe/Moscow

timedatectl status

HQ-RTR:

# Имя хоста:

hostnamectl hostname hq-rtr.au-team.irpo

exec bash

# Интерфейсы:

cd /etc/net/ifaces

mkdir enp7s1 enp7s2 enp7s2.100 enp7s2.200 enp7s2.999

echo TYPE=eth > enp7s1/options

echo TYPE=eth > enp7s2/options

echo TYPE=vlan HOST=enp7s2 VID=100 > enp7s2.100/options

echo TYPE=vlan HOST=enp7s2 VID=200 > enp7s2.200/options

echo TYPE=vlan HOST=enp7s2 VID=999 > enp7s2.999/options

echo 172.16.1.2/28 > enp7s1/ipv4address

echo 192.168.100.1/29 > enp7s2.100/ipv4address

echo 192.168.200.1/24 > enp7s2.200/ipv4address

echo 192.168.99.1/29 > enp7s2.999/ipv4address

echo default via 172.16.1.1 > enp7s1/ipv4route

echo nameserver 8.8.8.8 > enp7s1/resolv.conf

echo nameserver 8.8.8.8 > enp7s2.100/resolv.conf

echo nameserver 8.8.8.8 > enp7s2.200/resolv.conf

echo nameserver 8.8.8.8 > enp7s2.999/resolv.conf

systemctl restart network

# Настройка NAT

apt-get update

apt-get install -y nano

apt-get install -y iptables

nano /etc/net/sysctl.conf

net.ipv4.ip_forward = 1

sysctl -p /etc/net/sysctl.conf

iptables -t nat -A POSTROUTING -o enp7s1 -j MASQUERADE

iptables-save > /etc/sysconfig/iptables

systemctl enable --now iptables

# Пользователь net_admin

# https://stepik.org/lesson/2000243/step/1?unit=2028373

useradd -m net_admin

passwd net_admin

usermod -aG wheel net_admin

echo "net_admin ALL=(ALL:ALL) NOPASSWD:ALL" >> /etc/sudoers

# DHCP-сервер

# https://www.altlinux.org/DhcpBind

apt-get install -y dhcp-server

cd /etc/dhcp

cp dhcpd.conf.sample dhcpd.conf

nano dhcpd.conf

ddns-update-style none;

subnet 192.168.200.0 netmask 255.255.255.0 {

option routers                  192.168.200.1;

option subnet-mask              255.255.255.0;

option nis-domain               "au-team.irpo";

option domain-name              "au-team.irpo";

option domain-name-servers      192.168.100.2;

range dynamic-bootp 192.168.200.2 192.168.200.2;

default-lease-time 21600;

max-lease-time 43200;

}

systemctl enable --now dhcpd

# Часовой пояс

apt-get install -y tzdata

timedatectl set-timezone Europe/Moscow

timedatectl status

# GRE-туннель

https://www.altlinux.org/Strongswan#Создание_GRE-интерфейса_(RTR-1)

cd /etc/net/ifaces

mkdir gre1

echo "TYPE=iptun

TUNTYPE=gre

TUNLOCAL=172.16.1.2

TUNREMOTE=172.16.2.2

TUNTTL=64

TUNOPTIONS='ttl 64'" > gre1/options

echo 192.168.255.1/30 > gre1/ipv4address

systemctl restart network

# Динамическая маршрутизация OSPF

apt-get install -y frr

nano /etc/frr/daemons

ospfd=yes

systemctl enable --now frr.service

vtysh

conf t

router ospf

passive-interface default

network 192.168.100.0/29 area 0

network 192.168.200.0/24 area 0

network 192.168.99.0/29 area 0

network 192.168.255.0/30 area 0

exit

interface gre1

no ip ospf passive

ip ospf authentication-key 1234

end

wr mem

exit

# Замена адреса DNS на локальный

cd /etc/net/ifaces

echo nameserver 192.168.100.2 > enp7s1/resolv.conf

echo nameserver 192.168.100.2 > enp7s2.100/resolv.conf

echo nameserver 192.168.100.2 > enp7s2.200/resolv.conf

echo nameserver 192.168.100.2 > enp7s2.999/resolv.conf

systemctl restart network

HQ-SRV:

# Имя хоста:

hostnamectl hostname hq-srv.au-team.irpo

exec bash

# Интерфейсы:

cd /etc/net/ifaces

mkdir enp7s1 enp7s1.100

echo TYPE=eth > enp7s1/options

echo TYPE=vlan HOST=enp7s1 VID=100 > enp7s1.100/options

echo 192.168.100.2/29 > enp7s1.100/ipv4address

echo default via 192.168.100.1 > enp7s1.100/ipv4route

echo nameserver 8.8.8.8 > enp7s1.100/resolv.conf

systemctl restart network

# DNS-сервер

# https://www.altlinux.org/DhcpBind

apt-get update

apt-get install -y nano

apt-get install -y dnsmasq

nano /etc/dnsmasq.conf

listen-address=192.168.100.2

server=8.8.8.8

# HQ-RTR

address=/hq-rtr.au-team.irpo/172.16.1.2

ptr-record=2.1.16.172.in-addr.arpa,hq-rtr.au-team.irpo

# BR-RTR

address=/br-rtr.au-team.irpo/172.16.2.2

# HQ-SRV

address=/hq-srv.au-team.irpo/192.168.100.2

ptr-record=2.100.168.192.in-addr.arpa,hq-srv.au-team.irpo

# HQ-CLI

address=/hq-cli.au-team.irpo/192.168.200.2

ptr-record=2.200.168.192.in-addr.arpa,hq-cli.au-team.irpo

# BR-SRV

address=/br-srv.au-team.irpo/192.168.3.2

# ISP

address=/docker.au-team.irpo/172.16.1.1

address=/web.au-team.irpo/172.16.2.1

systemctl enable --now dnsmasq

# Пользователь sshuser

# https://stepik.org/lesson/2000243/step/1?unit=2028373

useradd -u 2026 -m sshuser

passwd sshuser

usermod -aG wheel sshuser

echo "sshuser ALL=(ALL:ALL) NOPASSWD:ALL" >> /etc/sudoers

# SSH-сервер

apt-get install -y openssh-server

nano /etc/openssh/sshd_config

Port 2026

AllowUsers sshuser

MaxAuthTries 2

Banner /etc/openssh/banner.txt

echo Authorized access only > /etc/openssh/banner.txt

systemctl enable --now sshd

# Часовой пояс

apt-get update

apt-get install tzdata -y

timedatectl set-timezone Europe/Moscow

timedatectl status

HQ-CLI:

# Имя хоста:

hostnamectl hostname hq-cli.au-team.irpo

exec bash

# Интерфейсы:

В графической среде:

Сетевые подключения –> параметры подключений –> Добавить –> Тип «VLAN»

- Родительский интерфейс enp7s1

- Идентификатор VLAN 200

- Имя интерфейса enp7s1.200

Выключить / включить поддержку сети

# Часовой пояс

apt-get update

apt-get install tzdata -y

timedatectl set-timezone Europe/Moscow

timedatectl status

BR-RTR:

# Имя хоста

hostnamectl hostname br-rtr.au-team.irpo

exec bash

# Интерфейсы

cd /etc/net/ifaces

mkdir enp7s1 enp7s2

echo TYPE=eth > enp7s1/options

echo TYPE=eth > enp7s2/options

echo 172.16.2.2/28 > enp7s1/ipv4address

echo 192.168.3.1/29 > enp7s2/ipv4address

echo default via 172.16.2.1 > enp7s1/ipv4route

echo nameserver 8.8.8.8 > enp7s1/resolv.conf

echo nameserver 8.8.8.8 > enp7s2/resolv.conf

systemctl restart network

# Настройка NAT

apt-get update

apt-get install -y nano

apt-get install -y iptables

nano /etc/net/sysctl.conf

net.ipv4.ip_forward = 1

sysctl -p /etc/net/sysctl.conf

iptables -t nat -A POSTROUTING -o enp7s1 -j MASQUERADE

iptables-save > /etc/sysconfig/iptables

systemctl enable --now iptables

# Пользователь net_admin

# https://stepik.org/lesson/2000243/step/1?unit=2028373

useradd -m net_admin

passwd net_admin

usermod -aG wheel net_admin

echo "net_admin ALL=(ALL:ALL) NOPASSWD:ALL" >> /etc/sudoers

# Часовой пояс

apt-get update

apt-get install tzdata -y

timedatectl set-timezone Europe/Moscow

timedatectl status

# GRE-туннель

https://www.altlinux.org/Strongswan#Создание_GRE-интерфейса_(RTR-1)

cd /etc/net/ifaces

mkdir gre1

echo "TYPE=iptun

TUNTYPE=gre

TUNLOCAL=172.16.2.2

TUNREMOTE=172.16.1.2

TUNTTL=64

TUNOPTIONS='ttl 64'" > gre1/options

echo "192.168.255.2/30" > gre1/ipv4address

systemctl restart network

# Динамическая маршрутизация OSPF

apt-get install frr -y

nano /etc/frr/daemons

ospfd=yes

systemctl enable --now frr.service

vtysh

conf t

router ospf

passive-interface default

network 192.168.3.0/29 area 0

network 192.168.255.0/30 area 0

exit

interface gre1

no ip ospf passive

ip ospf authentication-key 1234

end

wr mem

exit

# Замена адреса DNS на локальный

cd /etc/net/ifaces

echo nameserver 192.168.100.2 > enp7s1/resolv.conf

echo nameserver 192.168.100.2 > enp7s2/resolv.conf

systemctl restart network

BR-SRV:

# Имя хоста

hostnamectl hostname br-srv.au-team.irpo

exec bash

# Интерфейсы

cd /etc/net/ifaces

mkdir enp7s1

echo TYPE=eth > enp7s1/options

echo 192.168.3.2/29 > enp7s1/ipv4address

echo default via 192.168.3.1 > enp7s1/ipv4route

echo nameserver 8.8.8.8 > enp7s1/resolv.conf

systemctl restart network

# Пользователь sshuser

# https://stepik.org/lesson/2000243/step/1?unit=2028373

useradd -u 2026 -m sshuser

passwd sshuser

usermod -aG wheel sshuser

echo "sshuser ALL=(ALL:ALL) NOPASSWD:ALL" >> /etc/sudoers

# SSH-сервер

apt-get update

apt-get install -y nano

apt-get install -y openssh-server

nano /etc/openssh/sshd_config

Port 2026

AllowUsers sshuser

MaxAuthTries 2

Banner /etc/openssh/banner.txt

echo Authorized access only > /etc/openssh/banner.txt

systemctl enable --now sshd

# Часовой пояс

apt-get update

apt-get install tzdata -y

timedatectl set-timezone Europe/Moscow

timedatectl status

# Замена адреса DNS на локальный

cd /etc/net/ifaces

echo nameserver 192.168.100.2 > enp7s1/resolv.conf

systemctl restart network